Project: develop a potentiometric biosensor platform for extracellular histone detection whose performance is enhanced through Deep Learning techniques.

Experiments:

Method: Surface plasmon resonance (SPR)

Materials: Aptamer (KU7), 6-mercaptohexanol (MCH), Calf thymus histone (CTH), human histones (H3.2 and H4), HBS-EP+ buffer (0.1 M HEPES, 1.5 M NaCl, 0.03 M EDTA and 0.5% v/v Surfactant P20)

Purpose: Validate the binding between the purchased aptamer and histones. Compare measured association and dissociation rate constants (ka and kd) with published values.

| Analyte | Ligand | ka (M-1s-1) | kd (s-1) | KD |
| --- | --- | --- | --- | --- |
| KU7 | CTH | 1.62±0.03e5 | 9.15±0.06e-4 | 5.6±0.1nM |
| KU7 | H4 | 5.62±0.05e5 | 2.25±0.01e-3 | 4.01±0.04nM |

This work used the histone as the immobilized ligand and the aptamer as the analyte in solution. We are looking to create the reverse system.

Source: K. T. Urak et al., “RNA inhibitors of nuclear proteins responsible for multiple organ dysfunction syndrome,” Nature Communications, vol. 10, no. 1, p. 116, Jan. 2019, doi: 10.1038/s41467-018-08030-y.

CTH

Date: October 5-8th, 2020

File name when received: 201008_851bew075

Human Histones

Date: December 3rd, 2020

File name when received: 201203_851bew104

SPR Files

- There are two folders in the “SPR Data Files” folder: one for CTH and one for human histones.

- PowerPoint "20200828_Biosensor_SPR_Project" was background information submitted to the SPR technician.

- “210402_Spyros_Cycle_Legend” is an Excel sheet prepared by the technician after the two experiments that outline the immobilization injections and the concentrations (confirmed by NanoDrop spectrophotometry).

Text Files

- To import the downloaded .txt. file into Excel, open a new sheet and go to “Data.” Import the data “From Text” as tab delimited to create a pivot table.

- The column headers are named by the SPR instrument.

- For example:

  - Cycle=1_Fc=3_X

    - Cycle 1, according to the Cycle Legend sheet

    - Flow cell 3, which has a monolayer based on the immobilization table in the Cycle Legend.

      - In the CTH experiment, FC3 is the control surface made of just MCH, the short carbon chain blocking molecule.

      - In the human histones experiment, FC3 is an active surface with 15 hours of aptamer and 5 hours of MCH.

    - X indicates the x-axis of the sensogram, which is time.

    - Y indicates Response Units (RU). This is the measured value in SPR, which is based off the angle in the incident light beam which is dependent on the mass of analyte interacting with the monolayer. This value is relative and is usually in the 10,000s.

  - Cycle=4_Fc=4-3_X and Cycle=4_Fc=4-3_Y

    - These columns are generated by the instrument and not its own flow cell. It is created by subtracting the RUs from the control flow cell from the active flow cell.

  - PowerPoint “Cycle 10 – CTH Experiment” graphically shows an example

- Concentrations were run from 0nM (buffer) to 200 or 400nM from lowest to highest.

- CTH experiment used “Multi Cycle” where the solution was flowed in series through the flow cells (FC3 then FC4) with buffer (dissociation) and regeneration chemicals run in between.

- Human histones were attempted as “Single Cycle” for the first 7 cycles where concentrations were run sequentially with buffer and regeneration only at the end. The data analysis in the related PowerPoint only considers the “Multi Cycle” data in cycles 8-45.

- Regeneration is not recorded as a separate injection cycle. It is the large peak after the 180s association injection, the 600s dissociation curve, and any time needed for instrument operation. The peak happens around the 900 to 1000s time.

  - CTH: 50 mM NaOH, 1 M NaCl, 60 sec @ 30 µL/min (30 sec baseline stabilization)

  - HH: 10 mM HCl, 60 sec @ 30 µL/min (60 sec baseline stabilization)